ESP-LXIVM Controller Specifications

        - The Rain BirdTM ESP-LXIVM Irrigation controller features and specifications include but are not limited to:

          - The controller shall be of a hybrid type that is microelectronic circuitry capable of fully automatic or manual operation.

          - The controller shall be housed in a wall-mountable, weather resistant plastic cabinet with a key-locking cabinet door suitable for indoor or outdoor installation.

          - The controller shall be a two-wire path field device based control system with integrated 2-wire device for valve operation (without separate decoder and solenoid devices)

          - The controller shall have incorporated a base station capacity of 60 stations (LX-IVM) or 240 stations (LX-IVM Pro).

          - LX-IVM can be upgraded to LX-IVM Pro with a LX-IVM Pro Panel.

          - All stations shall have the capability of independently obeying or ignoring a weather sensor as well as using or not using the master valve.

          - The weather sensor options shall include but are not limited to the Rain BirdTM WR2 Rain/Freeze Sensor, the Rain BirdTM RSD Rain Sensor, and the Rain BirdTM Anemometer with Rain BirdTM PT3002 pulse monitor.

          - The controller shall have 10 (LX-IVM) or 40 (LX-IVM Pro) separate and independent programs which can have different start times, start day cycles and station run times.

            - Each program shall have up to 8 start times per day for a total of 32 possible start times per day.

            - The programs shall be allowed to overlap operations based on user-defined settings which control the number of simultaneous stations per program and total for the controller.

          - The controller shall come standard with SimulStationsTM, a feature that allows the user to define up to 8 (LX-IVM) or 16 (LX-IVM Pro) Stations and any associated Master Valves to operate simultaneously.

          - The controller shall have a Cycle+SoakTM water management feature which is capable of operating each station for a maximum cycle time, and a minimum soak time to reduce water run-off.

          - The controller shall have a FloManagerTM feature that provides efficient irrigation and reduced water windows with real-time flow and station management.

          - The controller shall have FloWatchTM feature that learns the normal flow rate of each station and provides high and low flow detection.

          - The controller shall be compatible with the IQ v4.0 Central Control System utilizing IQ-NCC Network Communication Cartridges.

          - Shall include a 3-year trade warranty.

          - Shall be manufactured by Rain Bird® Corporation.

        - Use Rain BirdTM LXMM lockable powder-coated steel case suitable for wall mounting or free standing pedestal mounting.

        - Use Rain BirdTM LXMMPED powder-coated steel pedestal for pedestal mounting in conjunction with the Rain BirdTM LXMM case.

        - IVM accompanying field device specifications include, but are not limited to:

          - Shall have four different field device options to allow the user to choose the precise amount of landscape irrigation control needed.

            - Rain BirdTM LXIVMSOL interfacing signal line and Rain BirdTM valve.

            - Rain BirdTM LXIVMOUT interfacing signal line and Third Party Valve with compatible DC latching solenoid

            - Rain BirdTM LXIVMSEN interfacing signal line and compatible flow or weather sensor

            - Rain BirdTM LXIVMSD for surge protection. (every 500 ft or 15 field devices. whichever is shorter)